DM Plant – Simulation Backend Specification

1. Problem Description

An industrial Ion Exchange Demineralization (DM) plant modeled as a multi-stage flow shop system. Water enters in discrete batches and passes sequentially through five units. The simulation accounts for buffer capacities, processing times, resin exhaustion, regeneration, quality constraints, and stochastic failures.

2. System Layout

| Stage | Unit | Type |
| --- | --- | --- |
| W1 | Raw Water Tank | Buffer / Source |
| W2 | Cation Column | Processing + Capacity |
| W3 | Degasser | Processing |
| W4 | Anion Column | Processing + Capacity |
| W5 | Mixed Bed | Final Processing |

3. Arrival Process

| Parameter | Distribution / Value (Industry Range) |
| --- | --- |
| Interarrival time | Tarrival ∼ Exp(λ=0.05) per minute (mean 20 min) – typical for batch operation |
| Batch size | B ∼ Uniform(10,50) m³ – larger batches common in industrial DM |

4. Buffers

| Location | Capacity (m³) (Industry Range) |
| --- | --- |
| W1 tank | 1000 |
| Between W2/W3 | 200 |
| Between W3/W4 | 200 |
| Between W4/W5 | 200 |

5. Processing Times

| Unit | Processing Time | Notes |
| --- | --- | --- |
| W2 (Cation) | t2 = BQ2 with Q2 = 2.5 m³/min (150 m³/h) | Typical for medium DM |
| W3 (Degasser) | ∼Uniform(3,8) minutes | Realistic forced draft degasser |
| W4 (Anion) | t4 = BQ4 with Q4 = 2.5 m³/min | Same as cation |
| W5 (Mixed Bed) | t5 = BQ5 with Q5 = 2.0 m³/min | Slightly slower due to polishing |

6. Resin Capacity & Quality Constraints

| Unit | Resin Volume (m³) | Specific Capacity (eq/L) | Total Capacity (eq) |
| --- | --- | --- | --- |
| W2 | 50 | 1.2 | 60,000 |
| W4 | 40 | 1.0 | 40,000 |
| W5 | 30 | 0.8 | 24,000 |

Load per batch: hardness = 10 eq/m³ (typical for moderate feed water) → load = 10B eq. Quality model: hardness after W2 as before, with violation threshold 0.1 eq/m³. Regeneration triggers: capacity exhaustion OR hardness > 0.1 eq/m³.

7. Regeneration (Setup Time)

| Unit | Regeneration Time (minutes) (Industry Range) |
| --- | --- |
| W2 | Uniform(60,90) – typical 1-1.5 hours |
| W4 | Uniform(75,105) – anion slightly longer |
| W5 | Uniform(90,120) – mixed bed requires careful rinsing |

8. Machine Failures

| Failure Type | Distribution (Industry-Based) |
| --- | --- |
| Time to failure | Weibull(β=1.2, η=43200) minutes (MTBF ≈ 720 hours, typical for pumps/valves) |
| Repair time | Lognormal(μ=120, σ=30) minutes (mean 2 hours, reasonable for mechanical repairs) |

9. Decision Variables

- Flow rates Q₂, Q₄, Q₅

- Regeneration threshold (e.g., 90% of capacity)

- Inter-stage buffer sizes

10. Simulation Objectives

- Minimize total throughput time per batch

- Maximize equipment utilization

- Maintain water quality within limits

- Study sensitivity to failure rates and regeneration strategies

11. Process Flow Diagram

START

│

▼

┌─────────────────────────────────────────┐

│ GENERATE BATCH ARRIVAL │

│ - Interarrival time ~ Exp(λ) │

│ - Batch size B ~ Uniform(5,20) m³ │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ ADD TO W1 (RAW WATER TANK) │

│ - Update buffer level │

│ - Check overflow → FAIL │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ IS W2 (CATION) IDLE & W1 NOT EMPTY? │

└─────────────────────────────────────────┘

│

▼ (yes)

┌─────────────────────────────────────────┐

│ MOVE BATCH TO W2 │

│ - Start processing: t = B / Q2 │

│ - Update resin load: load += 5B │

│ - Schedule processing finish event │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ WHILE PROCESSING: │

│ - Check failure (Weibull) → if fail │

│ interrupt, go to REPAIR │

│ - Check load vs capacity or quality │

│ → if regen needed, go to REGEN │

└─────────────────────────────────────────┘

│

▼ (processing completes)

┌─────────────────────────────────────────┐

│ MOVE BATCH TO W2→W3 BUFFER │

│ - If buffer full → block W2 until space │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ IS W3 (DEGASSER) IDLE? │

└─────────────────────────────────────────┘

│

▼ (yes)

┌─────────────────────────────────────────┐

│ PROCESS IN W3: t ~ Uniform(2,5) min │

│ - Schedule finish event │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ MOVE BATCH TO W3→W4 BUFFER │

│ - Similar blocking rule │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ PROCESS IN W4 (ANION) │

│ - Same logic as W2 (load, capacity, │

│ regen, failures) │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ MOVE BATCH TO W4→W5 BUFFER │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ PROCESS IN W5 (MIXED BED) │

│ - Same logic + final quality check │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ BATCH COMPLETED → OUTPUT │

│ - Record statistics (throughput time, │

│ quality, etc.) │

└─────────────────────────────────────────┘

│

└──► (loop to next arrival)

┌─────────────────────────────────────┐

│ REGEN LOOP │

└─────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ UNIT ENTERS REGENERATION │

│ - Setup time ~ Uniform(90..240 min) │

│ - Unit unavailable │

│ - Upstream buffer builds queue │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ RESET RESIN LOAD │

│ - Unit returns to idle │

└─────────────────────────────────────────┘

│

└──► Resume normal processing

┌─────────────────────────────────────┐

│ FAILURE LOOP │

└─────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ UNIT FAILS (Weibull) │

│ - Current batch returned to upstream │

│ - Unit state = failed │

│ - Schedule repair finish event │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ REPAIR TIME ~ Lognormal(60,15) min │

│ - Unit returns to idle │

│ - Check upstream buffer │

└─────────────────────────────────────────┘

│

└──► Resume processing

Step-by-Step Walkthrough

1. Arrival Generation

The simulation starts by generating the first batch. Interarrival times are exponential (mean 10 min); batch sizes are uniform 5-20 m³.

2. Raw Water Tank (W1)

The batch enters W1, increasing its level. If the level exceeds 500 m³, an overflow failure occurs (system halt). Otherwise, the batch waits until the downstream unit (W2) is idle and W1 has volume.

3. Cation Column (W2)

When W2 is idle, the batch moves from W1 to W2. Processing time = batch size / flow rate (1.5 m³/min). Resin load is updated (hardness 5 eq/m³ → load += 5B).

While processing, two checks run in parallel:

- Failure check: Weibull distribution determines if a failure occurs. If yes, the batch is returned upstream, and the unit enters repair (Lognormal repair time).

- Regeneration check: if load ≥ capacity OR quality violation (hardness exceeds 0.1 eq/m³), the unit enters regeneration (setup time 90-240 min). After regeneration, load is reset.

Both checks interrupt the current batch.

4. Inter-stage Buffer (W2→W3)

After W2 finishes, the batch is placed into a 100 m³ buffer. If the buffer is full, W2 is blocked until space becomes available.

5. Degasser (W3)

When W3 is idle and the buffer has a batch, processing begins. Processing time is uniform 2-5 min (no resin or quality constraints). After completion, the batch moves to the next buffer (W3→W4).

5. Degasser (W3)

- When W3 is idle and the buffer has a batch, processing begins.

- Processing time is uniform 2–5 min (no resin or quality constraints).

- After completion, the batch moves to the next buffer (W3→W4).

6. Anion Column (W4)

- Same logic as W2: processing time = B / 1.5, load tracking, regeneration (120–150 min), failures.

- After W4, the batch enters the W4→W5 buffer.

7. Mixed Bed (W5)

- Processing time = B / 1.2, load tracking, regeneration (180–240 min), failures.

- Final quality check: if hardness exceeds limit, a quality violation is recorded.

8. Output & Statistics

- Once the batch exits W5, it is considered complete. Throughput time, quality, and other metrics are recorded.

- The simulation loops to generate the next batch arrival.

Key Decision Points

- Regeneration triggers: capacity exhaustion or quality violation.

- Blocking: prevents upstream processing if downstream buffer is full.

- Failures: handled as preemptive interrupts with repair.