RO Plant – Simulation Backend Specification

1. Problem Description

A Reverse Osmosis (RO) plant with parallel membrane trains. Water arrives in discrete batches, passes through pre-treatment, then is split into permeate and reject by one of the membrane trains. The simulation includes degradation (fouling), cleaning cycles, and stochastic failures.

2. System Layout

| Unit | Description | Type |
| --- | --- | --- |
| R1 | Feed Pump | Processing |
| R2 | Pre-treatment | Processing |
| R3 | RO Membrane | Parallel trains |
| R4 | Permeate Tank | Buffer |
| R5 | Reject Tank | Buffer |

3. Arrival Process

Parameter Distribution / Value (Industry Range)

- Batch size B ~ Uniform(20,100) m³ – typical RO skid batch sizes

- Interarrival time Poisson process with rate λ = 0.1 batches/min (mean 10 min)

4. Buffers

Tank Capacity (m³) (Industry Range)

- Feed (R1) 500

- Permeate (R4) 800

- Reject (R5) 400

5. Processing Times

Unit Processing Time

- Feed Pump 2 minutes (fixed)

- Pre-treatment 3 minutes (fixed) – may include dosing

- RO Membrane tRO = B / Qtrain with Qtrain = 10 m³/min per train (600 m³/h)

6. Flow Split & Degradation

Parameter Description / Value (Industry-Based)

- Initial efficiency η0 = 0.95

- Degradation rate Exponential: η(t) = η0 · exp(-k·V) with k = 0.00001 per m³ (fouling over months)

- Recovery R = 0.75 + 0.2·(η - 0.7) (typical recovery 70-85%)

- Permeate volume Vp = R·B

- Reject volume Vr = (1-R)·B

7. Cleaning (Setup)

Parameter Value (Industry Range)

- Trigger η < 0.7 (or ΔP > 2 bar)

- Cleaning time Uniform(90,180) minutes (1.5-3 hours)

- Cleaning interval Typically every 30-90 days; simulation triggers by efficiency

8. Failures

Failure Type Time to Failure (Industry MTBF) Repair Time (Industry)

- Pump Weibull(β=1.1, η=36000) minutes (MTBF ≈ 25,000 h) Lognormal(μ=180, σ=45) minutes (mean 3 h)

- Membrane clogging Weibull(β=1.5, η=86400) minutes (MTBF ≈ 60,000 h) Lognormal(μ=300, σ=60) minutes (mean 5 h)

9. Decision Variables

- Number of membrane trains

- Cleaning threshold ηmin

- Feed flow distribution among trains

10. Simulation Objectives

- Maximize permeate output while respecting quality

- Minimize energy consumption (pump runtime)

- Balance cleaning frequency vs. downtime

11. Process Flow Diagram

START

│

▼

┌─────────────────────────────────────────┐

│ GENERATE BATCH ARRIVAL                  │

│ - Batch size B ~ Uniform(10,50) m³      │

│ - Interarrival ~ Poisson(λ=0.2)         │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ ADD TO R1 (FEED TANK)                   │

│ - Check overflow → FAIL                 │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ MOVE BATCH THROUGH R2 (PRE-TREATMENT)   │

│ - Fixed processing time = 2 min         │

│ - If failure, go to FAILURE LOOP        │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ SELECT MEMBRANE TRAIN (RO)              │

│ - Round-robin or least-loaded           │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ PROCESS IN SELECTED TRAIN               │

│ - Processing time = B / Q_train         │

│ - Update degradation η = η - k*B        │

│ - Current recovery R = f(η)             │

│ - Split: permeate = R*B, reject=(1-R)*B │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ AFTER PROCESSING:                       │

│ - Send permeate to R4 (permeate tank)   │

│ - Send reject to R5 (reject tank)       │

│ - Check tank capacities → block if full │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ CHECK η AFTER BATCH                     │

│ - If η < η_min → trigger cleaning       │

└─────────────────────────────────────────┘

│

└──► (loop to next arrival)

┌─────────────────────────────────────┐

│         CLEANING LOOP               │

└─────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ MEMBRANE TRAIN ENTERS CLEANING          │

│ - Cleaning time ~ Uniform(60,120) min   │

│ - Train offline                         │

│ - Other trains handle load              │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ RESET η TO INITIAL VALUE                │

│ - Train returns to available pool       │

└─────────────────────────────────────────┘

│

└──► Resume normal

┌─────────────────────────────────────┐

│         FAILURE LOOP                │

│ (pump or membrane)                  │

└─────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ UNIT FAILS                              │

│ - Current batch returned upstream       │

│ - Schedule repair (Lognormal)           │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ REPAIR COMPLETE                         │

│ - Unit idle                             │

└─────────────────────────────────────────┘

│

└──► Resume processing

The RO plant uses parallel membrane trains. Batches flow through pre-treatment, then to a selected train where water is split into permeate and reject. Degradation (fouling) reduces efficiency over time, triggering cleaning cycles. Failures can occur in pumps or membranes.

Step-by-Step Walkthrough

Arrival Generation

- Batches arrive according to a Poisson process (λ = 0.2 batches/min).

- Batch size uniform 10-50 m³.

Feed Tank (R1)

- Batch is stored in R1 (200 m³ capacity). Overflow triggers system failure.

Pre-treatment (R2)

- Fixed 2-minute processing.

- Failure possible (pump failure) – if occurs, the batch is returned upstream, and the unit is repaired (Lognormal).

Membrane Train Selection

- A train is selected (round-robin or least-loaded).

- If all trains are busy, the batch waits in R1.

Processing in Selected Train

- Processing time = batch size / train flow rate (e.g., 5 m³/min).

- Degradation: efficiency η decreases by k·B (k = 0.0001).

- Recovery R = 0.7 + 0.2η (linear mapping).

- Permeate volume = R·B, reject volume = (1-R)·B.

Split to Tanks

- Permeate is sent to R4 (300 m³ capacity), reject to R5 (150 m³).

- If either tank is full, the upstream train is blocked until space is available.

Cleaning Check

- After each batch, η is compared to ηmin = 0.5.

- If η < ηmin, the train enters cleaning (60-120 min).

- During cleaning, the train is offline; other trains handle new batches.

- After cleaning, η is reset to 1.0.

Failures

- Separate failure models for pumps (Weibull β=1.2, η=720) and membranes (Weibull β=1.8, η=1440).

- When a unit fails, the current batch is returned upstream, and the unit is repaired (Lognormal).

- After repair, the unit resumes.

Output & Statistics

Completed batches contribute to permeate volume, energy consumption, and efficiency metrics.

Key Decision Points

- Cleaning trigger: η falls below threshold.

- Train selection: load balancing across parallel units.

- Failures: independent for each train and pump.