STP – Simulation Backend Specification

1. Problem Description

A modular Sewage Treatment Plant (STP) with biological and physical stages. Sewage arrives in batches (or time-varying flow) and is treated in an equalisation tank, an aeration tank (with biological reaction), a clarifier, and sludge handling. The simulation includes continuous-time reaction kinetics, sludge removal, and quality constraints.

2. System Layout

| Unit | Function | Type |
| --- | --- | --- |
| S1 | Equalisation Tank | Buffer |
| S2 | Aeration Tank | Processing + Reaction |
| S3 | Clarifier | Processing |
| S4 | Sludge Tank | Buffer + Removal |
| S5 | Output Tank | Buffer |

3. Arrival Process

Two modes (select one):

| Mode | Description |
| --- | --- |
| A | Time-dependent inflow: piecewise constant flow rates |
| B | Discrete batches: B ~ Normal(50,10) m³, interarrival ~ Exp(λ=0.05) min |

4. Buffers

| Tank | Capacity (m³) |
| --- | --- |
| S1 | 1000 |
| S4 | 300 |
| S5 | 500 |

Overflow: If S1 level + incoming batch > 1000 m³ → system failure (spillage).

Blocking: If S4 is full during sludge handling, upstream S3 may block.

5. Processing Times

| Unit | Processing Time |
| --- | --- |
| Aeration | Uniform(2,6) hours |
| Clarifier | Uniform(1,2) hours |

6. Reaction Model (Aeration Tank)

Continuous-time dynamics for BOD:

dC/dt = (Q/V)(C_in – C) – k·C
where k = 0.1 h⁻¹ (example).

Simulation uses an ODE solver (e.g., Euler integration) at each event or fixed time steps.

7. Sludge Handling

| Parameter | Value |
| --- | --- |
| Trigger | S4 level > 80% of 300 m³ (i.e., > 240 m³) |
| Removal time | Uniform(30,60) minutes |
| During removal | S4 unavailable; upstream S3 may block |

8. Failures

| Failure Type | Time to Failure | Repair Time |
| --- | --- | --- |
| Aerator | Weibull(β=1.1, η=960) minutes | Lognormal(μ=45, σ=15) minutes |
| Pump | Weibull(β=1.2, η=1440) minutes | Lognormal(μ=30, σ=10) minutes |

9. Constraints

| Constraint | Condition |
| --- | --- |
| Overflow | S1 level + incoming batch > 1000 m³ → system halts |
| Quality violation | Effluent BOD > 20 mg/L or TSS > 30 mg/L for > 15 minutes |

10. Decision Variables

- Aeration rate (DO)

- Sludge recycle ratio

- Buffer capacities (design)

11. Simulation Objectives

- Meet effluent standards

- Minimize energy consumption (aeration, pumping)

- Reduce sludge handling frequency

12. Process Flow Diagram

START

│

▼

┌─────────────────────────────────────────┐

│ GENERATE INFLOW                         │

│ - Time-dependent pattern OR             │

│   discrete batch: B ~ Normal(50,10) m³  │

│ - Interarrival ~ Exp(λ=0.05)            │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ ADD TO S1 (EQUALIZATION TANK)           │

│ - Update level                          │

│ - If level > 1000 m³ → OVERFLOW FAIL    │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ CHECK IF S2 (AERATION) CAN ACCEPT       │

│ - S2 not full & S1 has volume           │

└─────────────────────────────────────────┘

│

▼ (yes)

┌─────────────────────────────────────────┐

│ MOVE BATCH TO S2                        │

│ - Start aeration for duration ~ 2-6 hr  │

│ - ODE solver runs continuously          │

│   (BOD, MLSS dynamics)                  │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ AFTER AERATION:                         │

│ - Move batch to S3 (clarifier)          │

│ - Clarifier time ~ 1-2 hr               │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ SEPARATION:                             │

│ - Effluent → S5 (output tank)           │

│ - Sludge → S4 (sludge tank)             │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ CHECK S4 LEVEL                          │

│ - If > 80% of 300 m³ → trigger          │

│   sludge removal                        │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ CHECK S5 QUALITY (BOD, TSS)             │

│ - If exceed limits → VIOLATION           │

└─────────────────────────────────────────┘

│

└──► (loop to next inflow)

┌─────────────────────────────────────┐

│        SLUDGE REMOVAL LOOP          │

└─────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ SCHEDULE REMOVAL EVENT                  │

│ - Removal time ~ Uniform(30,60) min     │

│ - S4 unavailable during removal         │

│ - Upstream S3 may block                 │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ EMPTY S4 (reset level)                  │

│ - S4 returns to idle                    │

└─────────────────────────────────────────┘

│

└──► Resume normal

┌─────────────────────────────────────┐

│         FAILURE LOOP                │

│ (aerator, pump)                     │

└─────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ UNIT FAILS                              │

│ - Current batch interrupted             │

│ - Repair time (Lognormal)               │

└─────────────────────────────────────────┘

│

▼

┌─────────────────────────────────────────┐

│ REPAIR COMPLETE                         │

│ - Resume processing                     │

└─────────────────────────────────────────┘

│

└──► Continue

Step-by-Step Walkthrough

Inflow Generation

Two modes: (A) time-dependent piecewise constant flow, or (B) discrete batches (Normal 50±10 m³) with exponential interarrival.

Equalisation Tank (S1)

Inflow adds to S1 level (capacity 1000 m³). If level exceeds capacity, overflow failure occurs (spillage). Batches (or continuous flow) wait until the aeration tank (S2) can accept.

Aeration Tank (S2)

When S2 is ready, a batch is moved from S1. Processing time is uniform 2-6 hours, but during this period the ODE solver runs continuously:

dC/dt = (Q/V)(C_in – C) – k·C
This updates BOD and MLSS based on inflow, volume, and reaction rate k = 0.1 h⁻¹. Failures (aerator) can interrupt processing; repair follows Lognormal.

Clarifier (S3)

After aeration, the batch moves to S3 for 1-2 hours. Solids settle: effluent (treated water) goes to S5 (output tank), sludge goes to S4 (sludge tank).

Sludge Tank (S4)

S4 level increases with each batch. When level exceeds 80% of 300 m³ (240 m³), sludge removal is triggered. Removal takes 30-60 minutes; during this time S4 is unavailable and upstream units (S3) may block.

Output Tank (S5)

Effluent accumulates in S5 (500 m³ capacity). Quality is continuously monitored: BOD and TSS must remain below 20 mg/L and 30 mg/L respectively. If violations persist for >15 minutes, a quality alert is raised.

Failures

Aerator and pump failures are modeled with Weibull times to failure and Lognormal repairs. Failures interrupt current processing; batch may be delayed.

Loop

After batch completion, the next inflow event is scheduled.

Key Decision Points

- Sludge removal: triggered by level threshold.

- Quality violation: continuous monitoring with a duration threshold.

- Overflow: immediate system failure if S1 exceeds capacity.